# ENVIRONMENTAL REQUIREMENT – INADVERTENT DISCOVERY PROTOCOL

The Inadvertent Discovery Protocol (IDP) should be followed if cultural resources (older than 50 years old) and/or human remains are encountered during construction of Local Highway Technical Assistance Council (LHTAC) projects. In addition, please refer to the most recent edition of the Idaho Transportation Department (ITD) Standard Specifications for Highway Construction manual.

All items that could potentially be cultural resources or human remains are to be treated as if they are cultural resources and/or human remains until a clear determination is made by the Idaho Transportation Department, State Highway Archaeologist (ITD SHA).

The LHTAC Resident Engineer will be notified by the construction contractor that potential cultural resources have been identified during project construction activities.  The Engineer will then immediately notify the ITD SHA of any cultural resources and/or human remains or items that could potentially be cultural resources and/or human remains that have been identified.

In the event cultural resources or human remains are discovered within the project area, the construction contractor will implement the appropriate protocol outlined below:

CULTURAL RESOURCES

- In the event that cultural resources are discovered within the project area, at locations associated with the project, or planned for use on the project; all work within 50 feet in all directions will cease and the area will be cleared of all unnecessary personnel.  The construction contractor or ITD construction crews will secure the area.

- The construction contractor will immediately notify the associated LHTAC Engineer who will then contact the ITD SHA.

LHTAC Resident Engineer	Megan Kautz	work: 208-344-0565	cell 208-369-0066

LHTAC Resident Engineer	Matt Koster	work: 208-344-0565	cell 208-401-5473

LHTAC Cultural Resource       JoEllen Ross-Hauer	work: 208-344-0565	cell 986-285-9051

ITD SHA	Marc Münch	work 208-334-8449	cell 208-921-0368

•	ITD SHA will notify the State Historic Preservation Office and Native American Tribes (as appropriate).

•	All appropriate laws and policies will be followed to the best of construction contractor or LHTAC staff abilities.

HUMAN REMAINS

•	In the event that human remains with or without associated cultural resources are discovered within the project area, at locations associated with the project, or at locations planned for use on the project; work within 150 feet of the human remains will cease and the area will be cleared of all personnel other than one (1) or two (2) construction contractor or LHTAC staff member(s) who will stay with the human remains until the ITD SHA is notified. The construction contractor will secure the area and immediately notify the associated LHTAC Engineer who will then contact the ITD SHA who will contact the appropriate law enforcement personnel (if necessary).

•	ITD SHA will notify the State Historic Preservation Office and any Native American Tribes (as appropriate).

•	No photography of human remains is allowed.  This applies to cameras, cell phones, or any other device(s) having photo capabilities.

•	The human remains will be completely covered with a tarp or plain piece of cloth such as a rug, towel, or blanket. No new ground disturbance should occur if at all avoidable.

•	The human remains will not be touched, moved, or in any way caused to change position from that noted upon discovery.

•	All information related to the discovery will be held in strictest confidence.

•	All information related to the discovery known to contractor or staff will be provided to the

ITD SHA, and/or law enforcement.

•	All appropriate laws and policies will be followed to the best of construction contractor or LHTAC staff abilities.

In either case, the construction contractor will keep all information strictly confidential.  If information is shared with a contractor or subcontractor, that person will be fully informed about the confidentiality requirements and will agree to keep the information confidential. The ITD SHA will consult with appropriate parties to determine an appropriate course of action.

Proceeding with Construction

After an inadvertent discovery, some areas may be specified for close monitoring or ‘no work zones.’

Any such areas will be identified by the ITD SHA, and locations made available to the Project Manager and appropriate contractor personnel. Additional cultural resources investigations may be required.

*Separate protocols for the inadvertent discovery of human remains and/or cultural resources on ITD projects exist for projects within the Coeur d’Alene Reservation and Nez Perce Reservation and should be used in lieu of those identified here.

# ENVIRONMENTAL NOTE – STORM WATER

# This project is anticipated to have 1.25 acres of disturbance and has the potential to discharge to Waters of the United States, which requires Construction General Permit coverage.  With Engineer approval, a Low Erosivity Waiver (LEW) may be utilized in lieu of Construction General Permit coverage, if disturbances remain less than 5.0 acres and the calculated Rainfall Erosivity R factor is less than 5.0.  If approved, obtain a LEW. Include all LEW documentation in a Pollution Prevention Plan and retain on site.

# If anticipated construction time is extended and the Rainfall Erosivity R factor will exceed 5.0 the project will no longer be eligible for a LEW.  A SWPPP must be submitted for review and certification.  When certified a Notice of Intent must be filed with EPA or IPDES a minimum of 14 days prior to desired permit coverage.

# For R value calculations see:

# https://lew.epa.gov/

(INCLUDE FOR ALL PROJECTS THAT HAVE IDWR SUBMITTAL REQUIREMENTS OR OTHER UNIQUE PERMIT REQUIRMENTS, ALTER AS NECESSARY

ENVIRONMENTAL REQUIREMENT – STREAM ALTERATION PERMIT

This project has permit coverage under an Idaho Department of Water Resources, Stream Alteration permit.  Submit the approved dewatering plan to IDWR 14 days prior to the start of dewatering.  Notify IDWR XXXXX days prior to the start of construction at the following office:

Make submittals to:

file@idwr.idaho.gov

Aaron Golart aaron.golart@idwr.idaho.gov

North Idaho

josie.brown@idwr.idaho.gov

Stream Channel Protection Specialist

Idaho Department of Water Resources

Northern Regional Office

7600 N Mineral Drive, Suite 100

Coeur d’Alene, ID 83815-7763

South and East Idaho

Cass.Jones@idwr.idaho.gov and Katie.Gibble@idwr.idaho.gov

State Coordinator, Stream Protection Program 
Idaho Department of Water Resources 
322 E. Front St. 
P.O. Box 83720

Penalties and Damages

Fines, penalties, and costs to the Department for the Contractor’s failure to comply with the Clean Water Act, to mitigate environmental damage, or to resolve regulatory actions will be deducted from moneys due the Contractor.

# ENVIRONMENTAL REQUIREMENT – POLLINATOR PROTECTION

Implement the following Best Management Practices to support pollinators and pollinator habitat along roadside corridors:

•             Protect Existing Habitat: Protect existing stands of native vegetation. Ground disturbing activities will be limited only to those areas deemed necessary for the construction of the project.  Disturbing existing areas of native vegetation purely for the convenience of the contractor is prohibited.

•             Herbicide Use:  Reduce the risk of herbicide exposure to pollinators by:

(1) Eliminating or reducing herbicide exposure to pollinators by first utilizing non-chemical (manual) methods to eliminate noxious and undesirable weeds.

(2) If herbicide use is necessary, spot treat specific weeds with selective herbicides that do not leave residuals in the soil.

(3) Treat weeds before they flower, to avoid spraying when pollinators are present.

(4) Avoid spray application if winds are above 10 mph.

# ENVIRONMENTAL REQUIREMENT – (STORM WATER) POLLUTION PREVENTION PLAN

A Pollution Prevention Plan (PPP) ITD-2788 is required for this project.  If this project exceeds one acre or more of disturbance a Storm Water Pollution Prevention Plan (SWPPP) ITD-2950, and associated Idaho Pollution Discharge Elimination System Notice of Intent and Construction General Permit coverage will be required.  With engineer approval, in lieu of a Notice of Intent, a Low Erosivity Waiver (LEW) may be obtained for the project, if the total project disturbances are more than 1.0 acres but less than 5.0 acres and the Environmental Protection Agency calculated rainfall erosivity R factor is less than 5.0. Include all LEW documentation in the PPP and retain on site.

If anticipated construction time is extended and the Rainfall Erosivity R factor will exceed 5.0 the project will no longer be eligible for a LEW.  A SWPPP must be submitted for review and certification.  When certified a Notice of Intent must be filed and construction general permit coverage obtained no less than 14 days prior to desired permit coverage.

For R value calculations see:

https://lew.epa.gov/

# ENVIRONMENTAL REQUIREMENT – NON-REPORTING  404 NATIONWIDE PERMIT 14 (NWP 14)

This project has 404 permit coverage under a US Army Corps of Engineers non-reporting 404 Nationwide Permit 14 (NWP 14) and the associated Idaho Department of Environmental Quality 401 Certification.

Penalties and Damages

Fines, penalties, and costs to the Department for the Contractor’s failure to comply with the Clean Water Act, to mitigate environmental damage, or to resolve regulatory actions will be deducted from moneys due the Contractor.

# ENVIRONMENTAL REQUIREMENT – WETLAND PROTECTION

Wetlands are present within the project area. Non-compliance with the Clean Water Act may result in enforcement action by federal regulatory agencies. Disturbances to wetland areas not specifically designated on the plans is strictly prohibited. Discharge of pollutants (including sediments) to the wetland or adjacent riparian area is strictly prohibited.

ON PAGE 181  SUBSECTION 251.01 MIGRATORY BIRD ACT TREATY COMPLIAMCE

Add the following sentence before the first paragraph of 251.03:

No clearing or removal of vegetation and trees is allowed between [April 1st and August 15th] unless work is approved by the Engineer.  To request approval, submit a survey that verifies the absence of Migratory Bird that follows the conditions below.